SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- DEBUGGING & OPTIMISATION TASK

| Course Code: | CSA1223 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL3- DEBUGGING & OPTIMISATION TASK |
| Weightage: | 20% | Total Marks: | 100 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | K.YOGANAND | Reg.No.: | 192525173 |
| Department: | CSE(AI/ML) | Date: | 12/08/26 |

ANNEXURE A

- A program performing signed integer addition using 2’s complement produces incorrect results when adding large values (e.g., +120 and +50 in 8-bit representation). Debug the issue by analyzing the binary computation and identifying whether overflow has occurred. Propose a solution to correctly detect and handle overflow in such cases.

- A processor using a ripple carry adder shows significant delay in executing arithmetic operations in a real-time system. Debug the performance bottleneck by examining how carry propagates through each bit. Suggest an optimized solution using a carry look-ahead adder, and explain how it reduces delay.

- A multiplication unit implementing Booth’s algorithm gives incorrect results for certain negative operands. Debug the step-by-step execution of the algorithm, focusing on bit-pair checking, arithmetic shifts, and sign extension. Identify the possible source of error and suggest corrections to ensure accurate signed multiplication.

- An embedded system using the restoring division algorithm experiences inefficiency due to repeated restoration steps, leading to increased execution time. Debug the algorithm’s workflow and identify where unnecessary operations occur. Propose an optimized approach using the non-restoring division method, explaining how it improves performance.

- A scientific application using IEEE 754 single precision floating-point representation produces inaccurate results when adding very small and very large numbers. Debug the issue by analyzing exponent alignment, normalization, and rounding errors. Suggest optimization techniques, such as using double precision or algorithmic adjustments, to improve accuracy.

ANSWERS:

Q1. Signed Integer Addition using 2’s Complement – Overflow Detection

Problem identification:
In 8-bit signed 2’s complement, the representable range is −128 to +127. The values +120 and +50 are both positive, but their mathematical sum is +170, which is outside the 8-bit signed range. Therefore, an overflow occurs.

Binary computation:
+120 = 01111000
+50  = 00110010
                 --------
       10101010

The 8-bit result 10101010 has its most significant bit equal to 1, so it represents −86 in 2’s complement, not +170. Thus the result is incorrect as a signed 8-bit value because the required value cannot be represented.

Overflow detection:
For signed addition, overflow occurs when two operands have the same sign but the result has the opposite sign. Here both operands have sign bit 0, while the result has sign bit 1, the same sign but the result has the opposite sign. Here both operands have sign bit 0, while the result has sign bit so overflow is confirmed. Equivalently, overflow can be detected using:
Overflow = (A7 = B7) AND (R7 ≠ A7)

Corrected solution:
Before storing the result in an 8-bit signed variable, check for signed overflow. If overflow is detected, report it or use a wider data type such as 16-bit signed integer.

Example:
120 + 50 = 170, which should be stored as 00000000 10101010 in 16-bit representation.

Conclusion:
The error is not in binary addition itself; it is caused by exceeding the range of 8-bit signed 2’s complement. Proper overflow detection and a wider representation correctly handle the calculation.

Testing:
120 + 50 → overflow
100 + 20 → 120, no overflow
−100 + (−40) → overflow
60 + 50 → 110, no overflow

Q2. Ripple Carry Adder – Delay and Carry Look-Ahead Optimization

Problem identification:
A ripple carry adder (RCA) calculates each bit by waiting for the carry from the previous bit. In an n-bit RCA, the carry may propagate through all n stages. This makes the circuit slow for real-time arithmetic operations.

Carry propagation example:
For each full-adder stage:
Si = Ai XOR Bi XOR Ci
Ci+1 = AiBi + Ci(Ai XOR Bi)

If C0 changes, C1 must be produced before C2 can be produced, then C3, and so on. Therefore, the final sum depends on a chain of carry delays.

Root cause:
The major bottleneck is sequential carry propagation. The worst-case delay increases approximately with the number of bits.

Optimized solution – Carry Look-Ahead Adder (CLA):
A CLA computes carry signals in parallel using Generate and Propagate terms:
Gi = AiBi
Pi = Ai XOR Bi
Ci+1 = Gi + PiCi

For a 4-bit CLA:
C1 = G0 + P0C0
C2 = G1 + P1G0 + P1P0C0
C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0
C4 = G3 + P3G2 + P3P2G1 + P3P2P1G0 + P3P2P1P0C0

The carries are calculated directly instead of waiting for one stage after another.

Performance improvement:
CLA greatly reduces carry propagation delay and is therefore more suitable for high-speed processors and real-time arithmetic. It uses additional hardware, but the gain in speed is significant.

Testing:
Compare an RCA and CLA with the same input operands and verify that both produce identical sums and carry-out. Measure propagation delay; the CLA should provide a lower delay for larger bit widths.

Conclusion:
The RCA is functionally correct but slow because of ripple carry propagation. Replacing it with a CLA improves arithmetic speed by calculating carries in parallel.

Replacing it with a CLA improves arithmetic speed by calculating carries in parallel.

Q3. Booth’s Multiplication – Debugging Signed Negative Operands

Problem identification:
Booth’s algorithm performs signed binary multiplication efficiently by examining adjacent bits of the multiplier. Incorrect results for negative operands usually arise from incorrect bit-pair checking, logical instead of arithmetic shifting, or improper sign extension.

Correct bit-pair checking:
At every iteration, inspect (Q0, Q−1):
00 → no operation
11 → no operation
01 → A = A + M
10 → A = A − M

After the required operation, perform an arithmetic right shift on the combined register [A, Q, Q−1].

Important debugging points:
1. Q−1 must initially be 0.
2. The multiplier and multiplicand must use the same fixed bit width.
3. Negative values must be represented using 2’s complement.
4. A right shift of A must copy the sign bit into the new most significant position.
5. Q−1 must receive the old Q0 after each arithmetic shift.
6. The algorithm must execute exactly n iterations for an n-bit multiplication.

Likely source of error:
If a logical right shift is used, zeros are inserted at the left. For a negative value this destroys the sign information. Similarly, if negative operands are not sign-extended to the required width, the addition/subtraction becomes incorrect.

Correction:
Use arithmetic right shift for [A,Q,Q−1] and preserve the sign bit of A. Use 2’s complement subtraction:
A = A + (2’s complement of M)

Step-by-step control:
Initialize A = 0, Q = multiplier, M = multiplicand, Q−1 = 0 and Count = n.
Check Q0Q−1.
Perform add/subtract/no operation.
Arithmetic right shift [A,Q,Q−1].
Decrease Count.
Repeat until Count = 0.
The final product is contained in [A,Q].

Testing:
Test positive × positive, positive × negative, negative × positive, and negative × negative. Also test zero and boundary values. For example, (−5) × 3 must produce −15, while (−5) × (−3) must produce +15.

Conclusion:
The main corrections are proper bit-pair checking, fixed-width 2’s complement representation, arithmetic shifting, and correct sign extension. These ensure accurate signed multiplication.

Q4. Restoring Division – Optimization using Non-Restoring Division

Problem identification:
In restoring division, after subtracting the divisor from the partial remainder, the algorithm checks whether the result is negative. If it is negative, the divisor is added back to restore the previous remainder. This repeated restoration introduces extra arithmetic operations and increases execution time.

Restoring workflow:
1. Shift the partial remainder and quotient.
2. Subtract the divisor from the partial remainder.
3. If the result is non-negative, set quotient bit to 1.
4. If the result is negative, restore the previous remainder by adding the divisor and set quotient bit to 0.
5. Repeat for all bits.

Source of inefficiency:
Whenever subtraction produces a negative partial remainder, an additional addition operation is required immediately to restore it. These repeated add-back operations increase the number of arithmetic steps.

Optimized approach – Non-Restoring Division:
Non-restoring division avoids immediate restoration. Instead:
• If the current partial remainder is positive or zero, shift and subtract the divisor.
• If the current partial remainder is negative, shift and add the divisor.
• Set the quotient bit according to the sign of the partial remainder.
• After the final iteration, if the remainder is negative, perform one final correction by adding the divisor.

Basic rule:
R ≥ 0 → R = 2R − M
R < 0 → R = 2R + M

Advantages:
1. Avoids restoration after every unsuccessful subtraction.
2. Reduces the number of arithmetic operations.
3. Improves execution time.
4. Is suitable for hardware implementations where division speed matters.

Testing:
For each test case, verify:
Dividend = (Divisor × Quotient) + Remainder
and ensure the remainder has the correct sign/range. Compare the number of restoration operations in restoring division with non-restoring division.

Conclusion:
The repeated add-back operation is the major inefficiency in restoring division. Non-restoring division removes these repeated restorations and performs a correction only when necessary, improving performance while maintaining correct quotient and remainder.

Q5. IEEE 754 Single Precision – Inaccurate Addition of Very Small and Very Large Numbers

Problem identification:
IEEE 754 single precision uses 32 bits: 1 sign bit, 8 exponent bits, and 23 fraction bits. It provides limited precision. When a very small number is added to a very large number, exponent alignment may shift the small number's significant bits beyond the available precision. The small value can therefore be rounded away.

Main debugging stages:
1. Exponent alignment:
The number with the smaller exponent is shifted right until both exponents are equal. If the exponent difference is very large, most or all significant bits of the smaller number are lost.

2. Mantissa/significand addition:
The aligned significands are added or subtracted according to their signs.

3. Normalization:
The result is shifted so that the significand is in normalized form and the exponent is adjusted.

4. Rounding:
Because only 23 fraction bits are stored, extra bits are discarded according to the rounding rule. This can introduce a small error.

Example:
If a very large value is added to a value that is extremely small compared with it, the single-precision representation may produce the same stored result as the large value alone because the small value is below the available precision.

Root cause:
The issue is mainly finite precision and loss of significance during exponent alignment and rounding. It is not necessarily an arithmetic-programming bug.

Corrected/optimized solution:
1. Use IEEE 754 double precision when higher accuracy is required. Double precision provides a much larger significand and exponent range.
2. Use numerically stable algorithms and avoid subtracting nearly equal numbers when possible.
3. Reorder additions when summing many values, for example by adding values of similar magnitude first.
4. Use guard, round, and sticky bits correctly in hardware/software implementations.
5. Use compensated summation methods such as Kahan summation when repeated floating-point additions accumulate error.

Testing:
Test values with similar magnitudes and values with extremely different magnitudes. Compare single-precision and double-precision results against a higher-precision reference. Check whether the absolute and relative errors decrease when double precision is used.

Conclusion:
The inaccurate result is caused by finite IEEE 754 single-precision accuracy, especially exponent alignment, normalization, and rounding. Using double precision and numerically stable algorithms can signifi

Code of Conduct:

I ________________________________________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Identification of Errors / Bugs (20%) | Debugging Approach & Analysis (20%) | Correctness of Debugged Solution (25%) | Optimization Techniques Applied (20%) | Testing and Documentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Identification of Errors / Bugs | 20% | Accurately identifies all errors and issues with complete understanding of the problem(4) | Identifies most errors with minor omissions(3) | Identifies limited errors; misses key issues(2) | Unable to identify errors or misunderstands the problem(1) |
| Debugging Approach & Analysis | 20% | Uses effective debugging techniques with clear analysis and root cause identification(4) | Applies suitable debugging methods with minor gaps(3) | Uses basic debugging methods with limited analysis(2) | No systematic debugging approach followed(0) |
| Correctness of Debugged Solution | 25% | Provides completely corrected solution with accurate functionality and expected output(5) | Provides working solution with minor errors(4) | Partially correct solution with limited functionality(3) | Incorrect solution or fails to resolve errors(0) |
| Optimization Techniques Applied | 20% | Applies efficient optimization methods to improve performance, memory usage and quality(4) | Performs optimization with noticeable improvements(3) | Limited optimization with minimal improvements(2) | No optimization applied or inefficient implementation(0) |
| Testing and Documentation | 15% | Performs complete testing with proper validation and clear documentation(3) | Provides adequate testing and documentation with minor issues(2) | Limited testing and incomplete documentation(1) | No proper testing or documentation provided(0( |

ANSWER SHEET – DEBUGGING & OPTIMISATION TASK

Q1. Signed Integer Addition using 2’s Complement – Overflow Detection

Problem identification:
In 8-bit signed 2’s complement, the representable range is −128 to +127. The values +120 and +50 are both positive, but their mathematical sum is +170, which is outside the 8-bit signed range. Therefore, an overflow occurs.

Binary computation:
+120 = 01111000
+50  = 00110010
                 --------
       10101010

The 8-bit result 10101010 has its most significant bit equal to 1, so it represents −86 in 2’s complement, not +170. Thus the result is incorrect as a signed 8-bit value because the required value cannot be represented.

Overflow detection:
For signed addition, overflow occurs when two operands have the same sign but the result has the opposite sign. Here both operands have sign bit 0, while the result has sign bit 1, so overflow is confirmed. Equivalently, overflow can be detected using:
Overflow = (A7 = B7) AND (R7 ≠ A7)

Corrected solution:
Before storing the result in an 8-bit signed variable, check for signed overflow. If overflow is detected, report it or use a wider data type such as 16-bit signed integer.

Example:
120 + 50 = 170, which should be stored as 00000000 10101010 in 16-bit representation.

Conclusion:
The error is not in binary addition itself; it is caused by exceeding the range of 8-bit signed 2’s complement. Proper overflow detection and a wider representation correctly handle the calculation.

Testing:
120 + 50 → overflow
100 + 20 → 120, no overflow
−100 + (−40) → overflow
60 + 50 → 110, no overflow

Q2. Ripple Carry Adder – Delay and Carry Look-Ahead Optimization

Problem identification:
A ripple carry adder (RCA) calculates each bit by waiting for the carry from the previous bit. In an n-bit RCA, the carry may propagate through all n stages. This makes the circuit slow for real-time arithmetic operations.

Carry propagation example:
For each full-adder stage:
Si = Ai XOR Bi XOR Ci
Ci+1 = AiBi + Ci(Ai XOR Bi)

If C0 changes, C1 must be produced before C2 can be produced, then C3, and so on. Therefore, the final sum depends on a chain of carry delays.

Root cause:
The major bottleneck is sequential carry propagation. The worst-case delay increases approximately with the number of bits.

Optimized solution – Carry Look-Ahead Adder (CLA):
A CLA computes carry signals in parallel using Generate and Propagate terms:
Gi = AiBi
Pi = Ai XOR Bi
Ci+1 = Gi + PiCi

For a 4-bit CLA:
C1 = G0 + P0C0
C2 = G1 + P1G0 + P1P0C0
C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0
C4 = G3 + P3G2 + P3P2G1 + P3P2P1G0 + P3P2P1P0C0

The carries are calculated directly instead of waiting for one stage after another.

Performance improvement:
CLA greatly reduces carry propagation delay and is therefore more suitable for high-speed processors and real-time arithmetic. It uses additional hardware, but the gain in speed is significant.

Testing:
Compare an RCA and CLA with the same input operands and verify that both produce identical sums and carry-out. Measure propagation delay; the CLA should provide a lower delay for larger bit widths.

Conclusion:
The RCA is functionally correct but slow because of ripple carry propagation. Replacing it with a CLA improves arithmetic speed by calculating carries in parallel.

Q3. Booth’s Multiplication – Debugging Signed Negative Operands

Problem identification:
Booth’s algorithm performs signed binary multiplication efficiently by examining adjacent bits of the multiplier. Incorrect results for negative operands usually arise from incorrect bit-pair checking, logical instead of arithmetic shifting, or improper sign extension.

Correct bit-pair checking:
At every iteration, inspect (Q0, Q−1):
00 → no operation
11 → no operation
01 → A = A + M
10 → A = A − M

After the required operation, perform an arithmetic right shift on the combined register [A, Q, Q−1].

Important debugging points:
1. Q−1 must initially be 0.
2. The multiplier and multiplicand must use the same fixed bit width.
3. Negative values must be represented using 2’s complement.
4. A right shift of A must copy the sign bit into the new most significant position.
5. Q−1 must receive the old Q0 after each arithmetic shift.
6. The algorithm must execute exactly n iterations for an n-bit multiplication.

Likely source of error:
If a logical right shift is used, zeros are inserted at the left. For a negative value this destroys the sign information. Similarly, if negative operands are not sign-extended to the required width, the addition/subtraction becomes incorrect.

Correction:
Use arithmetic right shift for [A,Q,Q−1] and preserve the sign bit of A. Use 2’s complement subtraction:
A = A + (2’s complement of M)

Step-by-step control:
Initialize A = 0, Q = multiplier, M = multiplicand, Q−1 = 0 and Count = n.
Check Q0Q−1.
Perform add/subtract/no operation.
Arithmetic right shift [A,Q,Q−1].
Decrease Count.
Repeat until Count = 0.
The final product is contained in [A,Q].

Testing:
Test positive × positive, positive × negative, negative × positive, and negative × negative. Also test zero and boundary values. For example, (−5) × 3 must produce −15, while (−5) × (−3) must produce +15.

Conclusion:
The main corrections are proper bit-pair checking, fixed-width 2’s complement representation, arithmetic shifting, and correct sign extension. These ensure accurate signed multiplication.

Q4. Restoring Division – Optimization using Non-Restoring Division

Problem identification:
In restoring division, after subtracting the divisor from the partial remainder, the algorithm checks whether the result is negative. If it is negative, the divisor is added back to restore the previous remainder. This repeated restoration introduces extra arithmetic operations and increases execution time.

Restoring workflow:
1. Shift the partial remainder and quotient.
2. Subtract the divisor from the partial remainder.
3. If the result is non-negative, set quotient bit to 1.
4. If the result is negative, restore the previous remainder by adding the divisor and set quotient bit to 0.
5. Repeat for all bits.

Source of inefficiency:
Whenever subtraction produces a negative partial remainder, an additional addition operation is required immediately to restore it. These repeated add-back operations increase the number of arithmetic steps.

Optimized approach – Non-Restoring Division:
Non-restoring division avoids immediate restoration. Instead:
• If the current partial remainder is positive or zero, shift and subtract the divisor.
• If the current partial remainder is negative, shift and add the divisor.
• Set the quotient bit according to the sign of the partial remainder.
• After the final iteration, if the remainder is negative, perform one final correction by adding the divisor.

Basic rule:
R ≥ 0 → R = 2R − M
R < 0 → R = 2R + M

Advantages:
1. Avoids restoration after every unsuccessful subtraction.
2. Reduces the number of arithmetic operations.
3. Improves execution time.
4. Is suitable for hardware implementations where division speed matters.

Testing:
For each test case, verify:
Dividend = (Divisor × Quotient) + Remainder
and ensure the remainder has the correct sign/range. Compare the number of restoration operations in restoring division with non-restoring division.

Conclusion:
The repeated add-back operation is the major inefficiency in restoring division. Non-restoring division removes these repeated restorations and performs a correction only when necessary, improving performance while maintaining correct quotient and remainder.

Q5. IEEE 754 Single Precision – Inaccurate Addition of Very Small and Very Large Numbers

Problem identification:
IEEE 754 single precision uses 32 bits: 1 sign bit, 8 exponent bits, and 23 fraction bits. It provides limited precision. When a very small number is added to a very large number, exponent alignment may shift the small number's significant bits beyond the available precision. The small value can therefore be rounded away.

Main debugging stages:
1. Exponent alignment:
The number with the smaller exponent is shifted right until both exponents are equal. If the exponent difference is very large, most or all significant bits of the smaller number are lost.

2. Mantissa/significand addition:
The aligned significands are added or subtracted according to their signs.

3. Normalization:
The result is shifted so that the significand is in normalized form and the exponent is adjusted.

4. Rounding:
Because only 23 fraction bits are stored, extra bits are discarded according to the rounding rule. This can introduce a small error.

Example:
If a very large value is added to a value that is extremely small compared with it, the single-precision representation may produce the same stored result as the large value alone because the small value is below the available precision.

Root cause:
The issue is mainly finite precision and loss of significance during exponent alignment and rounding. It is not necessarily an arithmetic-programming bug.

Corrected/optimized solution:
1. Use IEEE 754 double precision when higher accuracy is required. Double precision provides a much larger significand and exponent range.
2. Use numerically stable algorithms and avoid subtracting nearly equal numbers when possible.
3. Reorder additions when summing many values, for example by adding values of similar magnitude first.
4. Use guard, round, and sticky bits correctly in hardware/software implementations.
5. Use compensated summation methods such as Kahan summation when repeated floating-point additions accumulate error.

Testing:
Test values with similar magnitudes and values with extremely different magnitudes. Compare single-precision and double-precision results against a higher-precision reference. Check whether the absolute and relative errors decrease when double precision is used.

Conclusion:
The inaccurate result is caused by finite IEEE 754 single-precision accuracy, especially exponent alignment, normalization, and rounding. Using double precision and numerically stable algorithms can significantly improve accuracy.