SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- ANALYTICAL PROBLEM SOLVING

| Course Code: | CSA1223 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL1- ANALYTICAL PROBLEM SOLVING |
| Weightage: | 45% | Total Marks: | 100 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | K.YOGANAND | Reg.No.: | 192525173 |
| Department: | CSE(AI/ML) | Date: | 12/08/26 |

ANNEXURE A

- A processor performs arithmetic operations on signed 8-bit integers using 2’s complement representation. Consider two numbers: +45 and −23. Analyze in detail how the system performs both addition and subtraction, including binary conversion, alignment of sign bits, and intermediate steps. Determine whether overflow occurs, and explain how the processor detects overflow and interprets the final result.

- A high-performance computing system replaces a ripple carry adder with a 4-bit carry look-ahead adder (CLA) to reduce computation delay. Given two binary inputs, analyze how generate and propagate signals are formed and how carries are computed in advance. Compare the time delay of CLA with ripple carry addition and justify, with reasoning, why CLA improves speed in modern processors.

- In a signed multiplication operation, a processor uses Booth’s algorithm to multiply two numbers, −6 and +5. Analyze the algorithm step-by-step, including the role of the accumulator, multiplier, and Booth bit, along with arithmetic shifts. Explain how the algorithm efficiently handles signed numbers and reduces the number of addition/subtraction operations compared to traditional multiplication.

- A processor is required to perform division of two binary numbers (e.g., 13 ÷ 3) using both restoring and non-restoring division algorithms. Analyze both methods step-by-step, showing intermediate remainders and quotient formation. Compare the two approaches in terms of number of steps, complexity, and efficiency, and explain why non-restoring division is often preferred in modern systems.

- A scientific application requires precise handling of real numbers using the IEEE 754 standard for floating-point representation. Given a decimal number (e.g., −13.25), analyze how it is represented in both single precision and double precision formats, including sign, exponent, and mantissa fields. Further, explain how floating-point addition is performed between two such numbers, highlighting issues like normalization, rounding, and precision loss.

1.ANSWER – Q1

1. Problem Understanding:

The processor uses signed 8-bit 2’s-complement integers. Therefore the range is −128 to +127. We must calculate +45 + (−23) and +45 − (−23), show binary conversion and sign extension, and check overflow.

2. Binary Conversion and Sign Alignment:

+45 = 0010 1101₂.

23 = 0001 0111₂, so −23 is obtained by complementing and adding 1: 1110 1001₂.

Both operands are already 8 bits, so no extra sign extension is required.

3. Addition: +45 + (−23):

0010 1101   (+45)

+1110 1001   (−23)

1 0001 0110

Discard the carry out of the 8th bit: 0001 0110₂ = +22₁₀.

The result is positive and lies within −128 to +127.

4. Subtraction: +45 − (−23):

Subtraction is converted into addition of the 2’s complement of the subtrahend: 45 − (−23) = 45 + 23.

−(−23) = +23 = 0001 0111₂.

0010 1101   (+45)

+0001 0111   (+23)

Final result: 0100 0100 = (68)₁₀.

5. Overflow Detection:

For addition, signed overflow occurs when two operands with the same sign produce a result with the opposite sign. In 45 + (−23), the signs are different, so overflow cannot occur.

For 45 − (−23), the effective addition is 45 + 23. Both are positive and the result 68 is also positive; since 68 ≤ 127, there is no overflow. Equivalently, the carry into the sign bit and carry out of the sign bit are equal.

6. Interpretation:

Final results: +45 + (−23) = +22 and +45 − (−23) = +68. The processor performs subtraction through 2’s-complement addition, and the signed overflow logic confirms that both results are valid 8-bit integers.

2.ANSWER – Q2

1. Problem Understanding:

A 4-bit Carry Look-Ahead Adder (CLA) reduces carry propagation delay by computing carries in parallel instead of waiting for each carry to ripple through the next full adder.

2. Example Inputs:

To demonstrate the method, take A = 1011₂ and B = 0110₂. Let C₀ = 0.

For each bit i, generate Pᵢ = Aᵢ XOR Bᵢ and Gᵢ = Aᵢ AND Bᵢ. The carry relation is Cᵢ₊₁ = Gᵢ + PᵢCᵢ.

3. Generate and Propagate Signals:

From least significant bit to most significant bit: P₀=1, G₀=0; P₁=1, G₁=1; P₂=1, G₂=0; P₃=1, G₃=0.

Thus every bit propagates an incoming carry, while bit 1 can generate a carry independently.

4. Carry Computation in Advance:

C₁ = G₀ + P₀C₀ = 0.

C₂ = G₁ + P₁G₀ + P₁P₀C₀ = 1.

C₃ = G₂ + P₂G₁ + P₂P₁G₀ + P₂P₁P₀C₀ = 1.

C₄ = G₃ + P₃G₂ + P₃P₂G₁ + P₃P₂P₁G₀ + P₃P₂P₁P₀C₀ = 1.

5. Sum Calculation:

Sᵢ = Pᵢ XOR Cᵢ. Therefore S₀=1, S₁=0, S₂=0, S₃=0.

Result = 0001₂ with final carry C₄=1,

so ,

the complete 5-bit result is 10001₂ = 17₁₀.

6. Delay Comparison and Interpretation:

In a ripple-carry adder, C₁ must be produced before C₂, then C₂ before C₃, and so on. The worst-case carry delay therefore grows approximately linearly with the number of bits: O(n).

A CLA forms carries using parallel AND/OR logic, giving much smaller carry delay; for a small 4-bit block it is approximately constant with respect to the four stages. Larger processors combine CLA blocks hierarchically.

Hence CLA improves speed because carry information is available earlier, reducing the critical path and allowing faster arithmetic operations. The trade-off is increased hardware and wiring complexity.

3.ANSWER – Q3

1. Problem Understanding:

Booth’s algorithm performs signed binary multiplication using 2’s complement. We multiply M = −6 by Q = +5. Use 4-bit operands and an extra Booth bit Q₋₁ = 0; the accumulator A is initially 0000.

2. Initial Values:

M = −6 = 1010₂ (4-bit 2’s complement).

−M = +6 = 0110₂.

Q = +5 = 0101₂.

A = 0000, Q₋₁ = 0, count = 4.

3. Booth Rules:

Inspect the pair (Q₀, Q₋₁): 01 → A = A + M; 10 → A = A − M; 00 or 11 → no arithmetic operation.

After the operation, perform an arithmetic right shift on the combined register [A,Q,Q₋₁], preserving the sign bit of A.

4. Iteration Table:

Iteration 1: Q₀Q₋₁ = 10 → A = 0000 − 1010 = 0110. Arithmetic right shift → A=0011, Q=0010, Q₋₁=1.

Iteration 2: Q₀Q₋₁ = 01 → A = 0011 + 1010 = 1101. Arithmetic right shift → A=1110, Q=1001, Q₋₁=0.

Iteration 3: Q₀Q₋₁ = 10 → A = 1110 − 1010 = 0100. Arithmetic right shift → A=0010, Q=0100, Q₋₁=1.

Iteration 4: Q₀Q₋₁ = 01 → A = 0010 + 1010 = 1100. Arithmetic right shift → A=1110, Q=0010, Q₋₁=0.

5. Final Result:

After four iterations, the product is the concatenation AQ = 1110 0010₂.

11100010₂ is a negative 8-bit 2’s-complement number.

Invert → 00011101, add 1 → 00011110 = 30.

Therefore the product is −30₁₀.

Check: (−6) × (+5) = −30.

6. Interpretation and Efficiency:

Booth’s algorithm handles signed values naturally because arithmetic shifts preserve the sign. It also reduces operations when the multiplier contains consecutive 1s: instead of adding the multiplicand for every 1 bit, a transition 01/10 represents a single add/subtract boundary. This reduces the number of arithmetic operations compared with ordinary shift-and-add multiplication.

4.ANSWER –

1. Problem Understanding:

We divide 13₁₀ by 3₁₀. In binary, dividend Q = 01101₂ and divisor M = 00011₂. Using five quotient bits, the expected result is quotient 0100₂ = 4 and remainder 00001₂ = 1.

2. Restoring Division:

Initialize A = 00000, Q = 01101, M = 00011. Repeat for 5 bits: shift left [A,Q], subtract M from A, and if A is negative restore it by adding M and place quotient bit 0; otherwise place quotient bit 1.

3. Restoring Steps:

Step 1: Shift → A=00001,  Q=11010.  A−M=−2,   so restore A=00001 and Q₀=0.

Step 2: Shift → A=00011,  Q=10100.  A−M=0,     so Q₀=1 and A=00000.

Step 3: Shift → A=00001,  Q=01000.  A−M=−2,   so restore A=00001 and Q₀=0.

Step 4: Shift → A=00011,  Q=10000.  A−M=0,     so Q₀=1 and A=00000.

Step 5: Shift → A=00001,  Q=00000.  A−M=−2,    so restore A=00001 and Q₀=0.

Final restoring result: Q = 00100₂ = 4, A = 00001₂ = 1.

4. Non-Restoring Division:

Initialize A=00000 and Q=01101. At each step, shift left [A,Q]. If the previous A was non-negative, subtract M; if it was negative, add M. Set the quotient bit to 1 when the new A is non-negative and 0 when it is negative. After the final step, if A is negative, add M once for final correction.

Step 1: Shift → A=00001; subtract M → A=−2, Q₀=0.

Step 2: Since A<0, shift and add M → A=−1, Q₀=0.

Step 3: Since A<0, shift and add M → A=+1, Q₀=1.

Step 4: Since A≥0, shift and subtract M → A=−1, Q₀=0.

Step 5: Since A<0, shift and add M → A=−1, Q₀=0; final correction A=A+M=1. The quotient bits form 00100₂ = 4.

Final non-restoring result: Q = 4, remainder = 1.

5. Comparison:

Both methods require one iteration per quotient bit, so for a fixed 5-bit dividend they use five main iterations. Restoring division may perform an extra restore addition whenever subtraction makes A negative.

Non-restoring division avoids immediate restoration and instead carries the negative partial remainder into the next cycle. This reduces corrective operations and is generally more efficient for hardware implementations, although the control logic must track the sign of A.

6. Interpretation:

13 ÷ 3 = 4 remainder 1. Check: 3×4 + 1 = 13. Non-restoring division is attractive in modern processors because it avoids repeated restore operations and can reduce average arithmetic activity and division latency.

5.ANSWER – Q5

1. Problem Understanding:

IEEE 754 represents real numbers using sign, biased exponent, and fraction (mantissa) fields. We represent −13.25 in single precision (32-bit) and double precision (64-bit), then explain floating-point addition.

2. Convert −13.25 to Binary:

13₁₀ = 1101₂ and 0.25₁₀ = 0.01₂. Therefore 13.25 = 1101.01₂.

Normalize: 1101.01₂ = 1.10101₂ × 2³. Since the number is negative, sign S=1.

3. IEEE 754 Single Precision:

Sign bit = 1.

Exponent = 3 + 127 = 130 = 10000010₂.

Fraction field stores the bits after the leading 1: 10101000000000000000000.

Therefore the 32-bit representation is 1 | 10000010 | 10101000000000000000000.

Hexadecimal form: C1540000₁₆.

4. IEEE 754 Double Precision:

Sign bit = 1.

Exponent = 3 + 1023 = 1026 = 10000000010₂.

Fraction field is 1010100000000000000000000000000000000000000000000000 followed by zeros to make 52 fraction bits.

Therefore the double representation is 1 | 10000000010 | 1010100000000000000000000000000000000000000000000000.

Hexadecimal form: C02A800000000000₁₆.

5. Floating-Point Addition Procedure:

Example: add −13.25 and +5.5. Convert both to normalized binary. −13.25 = −1.10101×2³ and +5.5 = +1.011×2².

First align exponents to 2³: +5.5 = +0.1011×2³. Then add the signed significands: −1.10101 + 0.10110 = −0.11111, so the intermediate result is −0.11111×2³.

Normalize: −0.11111×2³ = −1.1111×2² = −7.75.

The exponent and fraction are then encoded again in IEEE 754 format.

6. Normalization, Rounding and Precision Loss:

If the significand after addition has leading zeros, it is shifted left and the exponent is decreased until the normalized form 1. is obtained. If there are too many fraction bits, IEEE 754 rounding is applied; the common default is round-to-nearest, ties-to-even.

Very small differences between numbers can be lost when a smaller exponent is aligned with a much larger one. This is called precision loss or cancellation depending on the operation. Single precision has 24 bits of significand precision including the hidden leading 1, while double precision has 53 bits, so double precision generally provides much smaller rounding error.

7. Interpretation:

For −13.25, both single and double precision represent the value exactly because 0.25 is exactly representable in binary. Double precision provides a wider exponent range and more fraction bits, making it more suitable for scientific calculations where numerical precision is important.

Code of Conduct:

I __K.YOGANAND__________________________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding (20%) | Method Selection (20%) | Solution Process (25%) | Accuracy of Calculation (20%) | Interpretation of Results (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 | 4/4 | 4/4 | 5/5 | 4/4 | 3/3 |
| Q2 | 4/4 | 4/4 | 5/5 | 4/4 | 3/3 |
| Q3 | 4/4 | 4/4 | 5/5 | 4/4 | 3/3 |
| Q4 | 4/4 | 4/4 | 5/5 | 4/4 | 3/3 |
| Q5 | 4/4 | 4/4 | 5/5 | 4/4 | 3/3 |

RUBRICS FOR CALCULATION:

Each question is answered to the full 20-mark level: 4 marks Understanding + 4 Method Selection + 5 Solution Process + 4 Calculation Accuracy + 3 Interpretation = 20 marks.

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately(4) | Understands problem with minor gaps(3) | Partial understanding; misses key elements(2) | Misinterprets or unable to understand problem(1) |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification(4) | Selects correct method with minor errors(3) | Inappropriate or partially correct method(2) | Unable to select method(1) |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout(5) | Mostly correct steps with minor mistakes(4) | Disorganized steps; multiple errors(3) | No clear process(0) |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer(4) | Minor calculation errors(3) | Frequent errors; incorrect final answer(2) | No valid calculations(0) |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance(3) | Basic interpretation with minor omissions(2) | Limited or unclear interpretation(1) | No interpretation(0) |