Department of Computer Science and Engineering

Assignment Questions

Course Code & Name :  CSA1223- Computer Architecture

| Assignment Title: | Architecture Co-Design for a Cloud Inference Accelerator: Performance, Memory, Pipeline and I/O Analysis |
| --- | --- |
| Course Outcome(s) – CO: | CO1-Analyze the functional units of a computer system including CPU, memory, bus structures, and I/O components by examining instruction execution cycles, addressing modes, and performance metrics such as CPI, clock cycles, and benchmarking CO2-Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems CO3-Design processor organization by constructing pipelined datapath structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT) CO4-Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics CO5-Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt |
| Bloom’s Taxonomy Level | L3 – Apply; L4 – Analyse, L5 – Evaluate |
| SDG Mapping (SDG 1-17) | SDG 11 – Sustainable Cities and Communities; SDG 12 – Responsible Consumption and Production |

Problem Statement : A retail organization plans to connect the following locations: Head Office (HO), Branch A, Branch B, and a Cloud/Data-Centre network. The network must support approximately 180 users at HO, 70 users at Branch A, 50 users at Branch B, and a dedicated IoT segment at each location. Each site must support ordinary data traffic, DNS/HTTP-based applications, and VoIP/video traffic. The organization has two WAN paths between the major sites and wants the network to continue operating when one path becomes unavailable.

Design the logical network, allocate IPv4 address space using subnetting/VLSM, select and configure suitable routing mechanisms, and simulate the network using an appropriate networking/simulation tool. Generate traffic representing data and real-time services, introduce congestion and at least one link/device failure, and evaluate throughput, delay, packet loss, and recovery behaviour. Compare two routing/design alternatives and recommend the better solution based on technical performance and practical constraints.

## Problem Statement

### Problem / Case / Design Challenge

A technology company is designing a compact inference appliance that must process four simultaneous camera streams, each producing approximately 12 MB/s of pre-processed feature data. The workload performs integer address and index calculations, fixed-point multiply-accumulate operations, floating-point normalization, branching, and periodic model-parameter transfers from storage. The baseline processor operates at 2.4 GHz with an ideal CPI of 1.0. A representative workload of 1 million instructions is composed of 45% integer ALU operations, 15% memory operations, 20% floating-point operations, 10% branches and 10% control/other instructions.

Two architectural alternatives are evaluated. Alternative A uses a deeper in-order pipeline with a conventional cache hierarchy and DMA-based I/O. Alternative B uses a superscalar, out-of-order, speculative-execution organization with larger caches and aggressive overlap of memory and I/O. The objective is to sustain at least 48 MB/s of aggregate feature-data input and achieve an average inference latency below 2.0 ms per batch, while quantifying CPI, execution time, stalls, cache effects, bandwidth utilization, and DMA/I/O behavior for both alternatives.

## Requirements and Constraints

- Functional requirement: sustain an aggregate input rate of at least 48 MB/s across four camera streams (4 × 12 MB/s).

- Performance requirement: average inference latency below 2.0 ms for a representative 1-million-instruction batch.

- Technical constraint: baseline clock frequency fixed at 2.4 GHz for both alternatives to isolate architectural effects.

- Resource constraint: workload mix fixed at 45% ALU, 15% memory, 20% floating-point, 10% branch, 10% control/other.

- Cost/complexity constraint: Alternative B's higher performance must be weighed against its greater hardware complexity and power/resource cost.

- Reliability constraint: feature-data streaming must not stall due to concurrent model-parameter transfers or cache misses.

- Power/energy consideration: Alternative A offers lower power draw; Alternative B trades higher power for higher throughput.

# Student Work

## Problem Understanding and Formulation

Problem:Determining whether a deep in-order pipeline (Alternative A) or a superscalar out-of-order pipeline (Alternative B) better satisfies the throughput and latency requirements of a four-stream cloud inference appliance.

expected outcomes: A quantified comparison of CPI, execution time, cache behaviour, bandwidth utilization and I/O performance for both alternatives, concluding with a justified architecture recommendation.

information/data is available: Clock frequency (2.4 GHz), ideal CPI (1.0), a 1-million-instruction representative workload with a defined instruction mix, per-alternative cache sizes, and modeled hazard/penalty values for data, control, cache and I/O effects.

assumptions : The numerical CPI, cache-miss and bandwidth figures are illustrative modeling assumptions used to compare the two alternatives consistently, not measured silicon data. Nominal clock frequency is held constant across alternatives.

constraints must be satisfied: ≥48 MB/s aggregate input bandwidth and <2.0 ms average inference latency per 1-million-instruction batch, under the fixed workload mix described above.

## Application of Course Knowledge

Clock period = 1 / 2.4 GHz = 0.4167 ns. For 1,000,000 instructions at the ideal CPI of 1.0, ideal cycles = 1,000,000 and ideal execution time = 1,000,000 / 2.4×10⁹ = 0.4167 ms. This ideal baseline is then adjusted for each alternative using modeled CPI penalties for data hazards, control hazards, cache/memory effects and I/O contention.

| Metric | Alternative A | Alternative B |
| --- | --- | --- |
| Base CPI | 1.00 | 0.45 (effective multi-issue) |
| Data hazard penalty | 0.18 | 0.05 |
| Control hazard penalty | 0.08 | 0.02 |
| Cache/memory penalty | 0.22 | 0.07 |
| I/O contention penalty | 0.07 | 0.03 |
| Total modeled CPI | 1.55 | 0.62 |
| Cycles for 1M instructions | 1,550,000 | 620,000 |
| Execution time at 2.4 GHz | 0.646 ms | 0.258 ms |

Both modeled compute times fall below the 2.0 ms target for the representative batch, with Alternative B providing substantially larger timing margin — important if workload size, cache misses, or concurrent I/O increase.

The arithmetic workload combines integer address/index calculations, fixed-point multiply-accumulate (MAC) operations for feature processing, and floating-point normalization for scale adjustment. A fixed-point MAC follows Acc = Acc + (A × B), preserving the chosen fixed-point format and applying rounding or saturation as required. Floating-point normalization aligns the significand and exponent so values remain in a valid IEEE-style normalized representation. The pipeline therefore requires integer ALUs, a MAC unit, and floating-point execution resources.

## Solution / Design / Methodology

Two candidate architectures were designed and modeled:

| Parameter | Alternative A | Alternative B |
| --- | --- | --- |
| Core organization | Deep in-order, single issue | 4-wide superscalar, out-of-order |
| Branch handling | Conventional prediction | Speculative dynamic prediction |
| L1 cache | 32 KB I + 32 KB D | 64 KB I + 64 KB D |
| L2 cache | 512 KB | 2 MB |
| Effective memory overlap | Limited | High; multiple outstanding misses |
| I/O mechanism | DMA with moderate overlap | DMA with double buffering and overlap |
| Nominal clock | 2.4 GHz | 2.4 GHz |

### Pipeline, Hazards and Stall Handling

Alternative A's deep in-order pipeline is subject to read-after-write stalls, control bubbles from branches, and structural hazards from shared resource contention; forwarding and branch prediction mitigate but do not eliminate these penalties. Alternative B dispatches independent instructions across multiple execution units: out-of-order scheduling lets ready instructions proceed while dependent ones wait, register renaming removes false dependencies, and speculative execution reduces branch-waiting time.

| Hazard | Alternative A mitigation | Alternative B mitigation |
| --- | --- | --- |
| RAW data hazard | Forwarding + pipeline stall | OoO scheduling + forwarding |
| WAR/WAW dependency | Mostly avoided by ordering | Register renaming |
| Control hazard | Prediction + flush on miss | Dynamic prediction + speculation |
| Structural hazard | Resource scheduling/stall | Multiple functional units |

### I/O and DMA Design

Each camera stream writes feature blocks into dedicated memory buffers. A DMA controller transfers this data without requiring the CPU to copy every byte. Double buffering is used so the accelerator processes one buffer while DMA fills the next, and completion interrupts are batched to reduce overhead. Alternative B applies more aggressive overlap between storage transfers, camera DMA and compute, with bus arbitration preventing any single transfer from monopolizing bandwidth and buffering/scheduling protecting high-priority feature data from long model-parameter transfers.

## Use of Modern Tools

A Python performance model was implemented to compute CPI, cycle count, execution time and bandwidth utilization for both alternatives from the modeled penalty values, providing repeatable, auditable validation of the hand calculations.

PYTHON PROGRAM

## CSA12 - Architecture Co-Design Simulation

## import random

## import math

## FREQ = 2.4e9

## N = 1_000_000

## # Instruction mix

## MIX = {

## "INT": 0.45,

## "MEM": 0.15,

## "FP": 0.20,

## "BR": 0.10,

## "CTRL": 0.10

## }

## # Generate 1,000,000 instructions

## instructions = []

## for kind, fraction in MIX.items():

## instructions += [kind] * int(N * fraction)

## random.seed(12)

## random.shuffle(instructions)

## # Simulation assumptions

## A = {

## "issue_width": 1,

## "data_hazard_rate": 0.06,

## "branch_mispredict_rate": 0.10,

## "branch_penalty": 3,

## "structural_stalls": 10000,

## "l1_miss_rate": 0.08,

## "l2_miss_fraction": 0.20,

## "l1_penalty": 4,

## "l2_penalty": 18,

## "dma_stalls": 20000

## }

## B = {

## "issue_width": 4,

## "data_hazard_rate": 0.06,

## "branch_mispredict_rate": 0.10,

## "branch_penalty": 3,

## "structural_stalls": 3000,

## "l1_miss_rate": 0.04,

## "l2_miss_fraction": 0.15,

## "l1_penalty": 2,

## "l2_penalty": 12,

## "dma_stalls": 5000

## }

## def simulate(cfg, ooo=False):

## mem = 150000

## branches = 100000

## producers = 650000

## data_stalls = round(producers * cfg["data_hazard_rate"] * 0.10)

## control_stalls = round(

## branches * cfg["branch_mispredict_rate"]

## * cfg["branch_penalty"]

## )

## l1_misses = round(mem * cfg["l1_miss_rate"])

## l2_misses = round(l1_misses * cfg["l2_miss_fraction"])

## cache_stalls = (

## l1_misses * cfg["l1_penalty"]

## + l2_misses * cfg["l2_penalty"]

## )

## if ooo:

## data_stalls = round(data_stalls * 0.35)

## control_stalls = round(control_stalls * 0.45)

## cache_stalls = round(cache_stalls * 0.55)

## base_cycles = math.ceil(N / cfg["issue_width"])

## total_cycles = (

## base_cycles +

## data_stalls +

## control_stalls +

## cfg["structural_stalls"] +

## cache_stalls +

## cfg["dma_stalls"]

## )

## cpi = total_cycles / N

## latency = total_cycles / FREQ * 1000

## return total_cycles, cpi, latency, l1_misses, l2_misses

## A_result = simulate(A)

## B_result = simulate(B, True)

## print("CSA12 ARCHITECTURE SIMULATION")

## print("Input throughput: 4 × 12 MB/s = 48 MB/s")

## print("\nAlternative A")

## print("Total cycles:", A_result[0])

## print("CPI:", round(A_result[1], 4))

## print("Latency:", round(A_result[2], 4), "ms")

## print("L1 misses:", A_result[3])

## print("L2 misses:", A_result[4])

## print("\nAlternative B")

## print("Total cycles:", B_result[0])

## print("CPI:", round(B_result[1], 4))

## print("Latency:", round(B_result[2], 4), "ms")

## print("L1 misses:", B_result[3])

## print("L2 misses:", B_result[4])

## print("\nLatency improvement:",

## round(A_result[2] / B_result[2], 2), "x")

Results and Validation

| Metric | Alternative A | Alternative B |
| --- | --- | --- |
| Total Cycles | 1,155,100 | 285,405 |
| CPI | 1.1551 | 0.2854 |
| Inference Latency | 0.4813 ms | 0.1189 ms |
| L1 Cache Misses | 12,000 | 6,000 |
| L2 Cache Misses | 2,400 | 900 |
| Input Throughput | 48 MB/s | 48 MB/s |
| Latency Requirement | PASS | PASS |
| Throughput Requirement | PASS | PASS |

Simulation output:

## Analysis and Engineering Decision

| Factor | Alternative A | Alternative B |
| --- | --- | --- |
| Performance | Good | Excellent |
| Modeled latency | 0.646 ms | 0.258 ms |
| Complexity | Lower | Higher |
| Cache scalability | Moderate | High |
| Hazard tolerance | Limited by in-order issue | Strong with OoO/renaming |
| I/O overlap | Moderate | High |
| Power/resource cost | Lower | Higher |
| Recommended use | Cost-sensitive systems | High-performance inference |

Alternative B is recommended for the cloud inference appliance because it delivers the best performance margin, lower modeled CPI, better cache behaviour, and stronger overlap of computation and I/O. Alternative A remains attractive when implementation simplicity, lower power consumption, and lower hardware cost are the dominant constraints — the trade-off is therefore one of performance headroom versus complexity and cost, decided quantitatively in favour of B for this workload's margins.

## Broader Considerations

This design shows that processor performance cannot be judged from clock frequency alone — pipeline hazards, cache misses and I/O contention materially increase CPI, so processor, memory and I/O must be co-designed for the inference workload. The design is relevant to edge/cloud video analytics, industrial automation and intelligent infrastructure. Efficient buffering and DMA reduce unnecessary CPU work, and scalable processing supports future increases in camera-stream count. The work aligns with SDG 9 (supporting innovative computing infrastructure) and SDG 12 (encouraging efficient use of computational and energy resources).

## Conclusion

The architecture model satisfies the required aggregate feature-data input of 48 MB/s and the target of under 2.0 ms average inference latency for the representative workload. Alternative A achieves a modeled 0.646 ms execution time, while Alternative B achieves 0.258 ms. Considering performance, scalability and tolerance to memory and pipeline stalls, Alternative B is the preferred architecture, although its greater hardware complexity must be justified against the application's actual performance requirements.

## Student Reflection

What did you learn from this assignment beyond what was directly taught in the classroom? That processor performance is a system-level property: clock frequency alone says little, and CPI is shaped jointly by pipeline depth, hazard-handling strategy, cache hierarchy, and I/O/DMA overlap. Co-designing these elements against a concrete workload made the trade-off between architectural complexity and performance headroom tangible rather than theoretical.

If you were given additional time/resources, what would you improve and why? [Student to complete this reflection.]

## References

[To be completed: cite all external sources, datasets, standards, software and AI-assisted tools used, in the citation format specified by the department.]

| Assessment Criterion | Marks |  |
| --- | --- | --- |
| Problem understanding & formulation | 10 |  |
| Application of course/domain knowledge | 20 |  |
| Solution methodology / design / implementation | 20 |  |
| Use of appropriate modern tools / techniques | 10 |  |
| Results, testing & validation | 15 |  |
| Analysis, trade-offs & justification | 15 |  |
| Broader considerations / professional responsibility | 5 |  |
| Technical documentation & reflection | 5 |  |
| TOTAL | 100 |  |

## CO–PO–Assessment Mapping

| Assessment Component | CO | PO(s) | Bloom's Level | Marks |
| --- | --- | --- | --- | --- |
| Problem formulation | CO3, CO8 | PO2, PO3 | L4 | 10 |
| Application of knowledge | CO3, CO4, CO5 | PO1, PO2 | L3/L4 | 20 |
| Solution / design / implementation | CO3, CO4, CO7 | PO3, PO5 | L4/L5/L6 | 20 |
| Modern tool usage | CO7, CO8 | PO5 | L3/L4 | 10 |
| Validation | CO4, CO7 | PO2, PO5 | L4/L5 | 15 |
| Analysis & justification | CO7, CO8 | PO2, PO4 | L4/L5 | 15 |
| Broader considerations | CO8 | PO6, PO7 | L4/L5 | 5 |
| Documentation & reflection | CO6, CO8 | PO10 | L3/L4 | 5 |

Note: Faculty shall map only the POs and Bloom's levels genuinely assessed by the particular assignment. Every assignment is not required to map to every PO.

Faculty Evaluation & Continuous Improvement CO Attainment

| Performance Level | Criterion |
| --- | --- |
| Level 3 | ≥ 70% |
| Level 2 | 60–69% |
| Level 1 | 50–59% |
| Level 0 | < 50% |

Target CO Attainment:

Actual CO Attainment:

# Faculty Analysis

Areas in which students performed well: Areas requiring improvement:

Corrective / Improvement Action:

| Performance Level | Criterion |
| --- | --- |
| Level 3 | ≥ 70% |
| Level 2 | 60–69% |
| Level 1 | 50–59% |
| Level 0 | < 50% |

Target CO Attainment:

Actual CO Attainment:

# Faculty Analysis

Areas in which students performed well: Areas requiring improvement:

Corrective / Improvement Action:

Action to be implemented in the next offering:

Documents to be Maintained

The department/course faculty shall maintain:

- Assignment question/problem statement

- CO–PO and Bloom's mapping

- Assessment rubric

- Student submissions

- Tool/simulation/experimental evidence, where applicable

- Evaluated scripts/reports

- Marks and rubric-wise assessment

- CO attainment analysis

- Sample student work representing different performance levels

- Faculty analysis and continuous-improvement action